🔌 STEP 2: HARDWARE SETUP

🧾 Components

- Raspberry Pi 4 Model B

- USB Webcam

- 16x2 I2C LCD

- 2 × Digital IR Sensors

- Breadboard + Jumper wires

📍 GPIO CONNECTIONS

🔹 I2C LCD

| LCD Pin | Pi Pin | BCM |
| --- | --- | --- |
| VCC | 5V | Pin 2 |
| GND | GND | Pin 6 |
| SDA | SDA | BCM 2 (Pin 3) |
| SCL | SCL | BCM 3 (Pin 5) |

🔹 IR Sensors

| Sensor | GPIO (BCM) | Physical Pin |
| --- | --- | --- |
| ISSUE | BCM 17 | Pin 11 |
| RETURN | BCM 27 | Pin 13 |

⚠️ Precautions

- Never connect 5V to GPIO directly

- Use proper ground common connection

- IR output must be 3.3V compatible

1. HOW BREADBOARD WORKS (IMPORTANT)

🔹 Internal Connections

Power Rails:
 + + + + → Connected horizontally
 - - - - → Connected horizontally

 Middle Section:
 | | | | → Connected vertically (each column)

👉 This means:

- All red line = 5V

- All blue line = GND

⚡ 2. FIRST STEP: POWER DISTRIBUTION (DO THIS FIRST)

🔹 Connect Raspberry Pi to Breadboard

Pi 5V (Pin 2) → Breadboard RED rail (+)
 Pi GND (Pin 6) → Breadboard BLUE rail (-)

✅ Now whole breadboard has:

- 5V supply

- Ground

📺 3. I2C LCD CONNECTION (WITH BREADBOARD)

🔹 Wiring

LCD VCC → Breadboard + (5V rail)
 LCD GND → Breadboard - (GND rail)

 LCD SDA → GPIO2 (Pin 3)
 LCD SCL → GPIO3 (Pin 5)

🔥 Important Tip

If LCD not working:

- Try address 0x27 or 0x3F

- Use i2cdetect -y 1

🔴 4. IR SENSOR CONNECTION (VERY IMPORTANT)

Each IR sensor has:

- VCC

- GND

- OUT

🔹 ISSUE SENSOR

VCC → Breadboard + (5V)
 GND → Breadboard - (GND)
 OUT → GPIO17 (Pin 11)

🔹 RETURN SENSOR

VCC → Breadboard + (5V)
 GND → Breadboard - (GND)
 OUT → GPIO27 (Pin 13)

⚠️ VERY IMPORTANT (VIVA POINT)

Most IR sensors output 3.3V safe signal, but:
 👉 If output is 5V → use voltage divider

🎥 5. USB CAMERA

Plug directly into Raspberry Pi USB port

✔ No breadboard needed

🧩 6. FINAL CONNECTION SUMMARY

Raspberry Pi
 │
 ├── 5V → Breadboard +
 ├── GND → Breadboard -
 │
 ├── GPIO2 → LCD SDA
 ├── GPIO3 → LCD SCL
 │
 ├── GPIO17 → IR ISSUE
 └── GPIO27 → IR RETURN

⚠️ COMMON MISTAKES (VERY IMPORTANT)

❌ Connecting LCD SDA to wrong pin
 ❌ Not connecting common GND
 ❌ IR OUT to 5V instead of GPIO
 ❌ Not powering breadboard

🔍 7. HOW TO TEST CONNECTIONS (STEP-BY-STEP)

🔹 Test IR Sensor

import RPi.GPIO as GPIO

 GPIO.setmode(GPIO.BCM)
 GPIO.setup(17, GPIO.IN)

 while True:
 	print(GPIO.input(17))

👉 Move hand → value should change

🔹 Test LCD

from RPLCD.i2c import CharLCD

 lcd = CharLCD('PCF8574', 0x27)
 lcd.write_string("Working!")

⚙️ STEP 3: RASPBERRY PI SETUP

sudo apt update && sudo apt upgrade -y

 sudo raspi-config

Enable:

- I2C

- Camera

- SSH

Reboot:

sudo reboot