# 📦 🧠 COMPLETE LIBRARY INSTALLATION (FROM ZERO)

We will install everything required for:

✅ Camera (OpenCV)
✅ Barcode scanning (pyzbar)
✅ Database (MySQL connector)
✅ LCD (I2C)
✅ GPIO (IR sensors)
✅ GUI (Tkinter)

# 🧱 STEP 0: PREPARE SYSTEM (VERY IMPORTANT)

Open terminal:

sudo apt update

sudo apt upgrade -y

## 🔴 Install Basic Tools

sudo apt install python3-pip python3-dev build-essential -y

👉 These prevent installation errors later

# 📷 STEP 1: INSTALL OPENCV (CAMERA)

## 🔹 Install via APT (RECOMMENDED — NO ERRORS)

sudo apt install python3-opencv -y

## 🔹 Test OpenCV

python3

import cv2

print(cv2.__version__)

👉 If no error → SUCCESS ✅

## ❌ If Error: "No module named cv2"

Fix:

pip3 install opencv-python

# 🔍 STEP 2: INSTALL BARCODE LIBRARY (PYZBAR)

## 🔹 Install Dependency FIRST (VERY IMPORTANT)

sudo apt install libzbar0 -y

👉 Without this → pyzbar will FAIL

## 🔹 Install pyzbar

pip3 install pyzbar

## 🔹 Test

from pyzbar.pyzbar import decode

👉 If no error → OK

# 🗄️ STEP 3: INSTALL MYSQL CONNECTOR

pip3 install mysql-connector-python

## 🔹 Test

import mysql.connector

## ❌ Error Fix

If error:

pip3 install --upgrade mysql-connector-python

# 🔌 STEP 4: INSTALL GPIO LIBRARY

## 🔹 Install

sudo apt install python3-rpi.gpio -y

## 🔹 Test

import RPi.GPIO as GPIO

# 📺 STEP 5: INSTALL LCD (I2C) LIBRARY

## 🔹 Install SMBUS (IMPORTANT)

sudo apt install python3-smbus i2c-tools -y

## 🔹 Install LCD Library

pip3 install RPLCD

## 🔹 Test I2C Device

i2cdetect -y 1

👉 Should show:

0x27  or 0x3F

# 🖥️ STEP 6: TKINTER (GUI)

## 🔹 Install

sudo apt install python3-tk -y

## 🔹 Test

import tkinter

# 🧪 STEP 7: FINAL TEST SCRIPT (VERY IMPORTANT)

Create file:

nano test_all.py

Paste:

import cv2

from pyzbar.pyzbar import decode

import mysql.connector

import RPi.GPIO as GPIO

from RPLCD.i2c import CharLCD

import tkinter

print("All Libraries Installed Successfully!")

Run:

python3 test_all.py

# ⚠️ COMMON ERRORS + FIXES (VERY IMPORTANT)

## ❌ 1. Camera Not Opening

Fix:

ls /dev/video0

If not found:

- Replug camera

- Try another USB port

## ❌ 2. pyzbar Error

ImportError: Unable to find zbar shared library

Fix:

sudo apt install libzbar0

## ❌ 3. I2C Not Working

Fix:

sudo raspi-config

Enable I2C

## ❌ 4. Permission Error (GPIO)

Run script using:

sudo python3 main.py

## ❌ 5. LCD Not Displaying

Fix:

- Check address (0x27 / 0x3F)

- Check wiring

# 🧠 INSTALLATION SUMMARY (SHORT)

sudo apt update && sudo apt upgrade -y

sudo apt install python3-pip python3-opencv libzbar0 python3-rpi.gpio python3-smbus i2c-tools python3-tk -y

pip3 install pyzbar mysql-connector-python RPLCD