# 🧠 📦 WHAT ARE MODULES?

👉 Instead of writing everything in one file, we divide code into parts (modules):

| Module | Purpose |
| --- | --- |
| sensors.py | IR sensor detection |
| scanner.py | Barcode scanning |
| lcd.py | LCD display |
| database.py | MySQL connection |
| main.py | Main logic (connects all) |

## 🔥 WHY USE MODULES? (VIVA GOLD)

Say this:

👉
"Modular programming improves readability, debugging, and scalability. Each module handles a specific task."

# 🧩 1. sensors.py (IR SENSOR MODULE)

## ✅ FULL CODE

import RPi.GPIO as GPIO

import time

ISSUE_PIN = 17

RETURN_PIN = 27

GPIO.setmode(GPIO.BCM)

GPIO.setup(ISSUE_PIN, GPIO.IN)

GPIO.setup(RETURN_PIN, GPIO.IN)

def detect_mode(timeout=10):

start = time.time()

while time.time() - start < timeout:

if GPIO.input(ISSUE_PIN) == 0:

time.sleep(0.3)  # debounce

return "ISSUE"

if GPIO.input(RETURN_PIN) == 0:

time.sleep(0.3)

return "RETURN"

return None

## 🔍 LINE-BY-LINE EXPLANATION

### 🔹 Import libraries

import RPi.GPIO as GPIO

import time

- RPi.GPIO → control Raspberry Pi pins

- time → used for delay & timeout

### 🔹 Define pins

ISSUE_PIN = 17

RETURN_PIN = 27

👉 These match your wiring

### 🔹 Setup GPIO

GPIO.setmode(GPIO.BCM)

👉 Use BCM numbering (IMPORTANT)

GPIO.setup(ISSUE_PIN, GPIO.IN)

👉 Set pin as INPUT

### 🔹 Main function

def detect_mode(timeout=10):

👉 Waits for user action

### 🔹 Timer logic

start = time.time()

👉 Prevent infinite loop

### 🔹 Detect ISSUE

if GPIO.input(ISSUE_PIN) == 0:

👉 IR sensor triggered

### 🔹 Debounce

time.sleep(0.3)

👉 Avoid false detection

### 🔹 Return result

return "ISSUE"

### 🔹 Timeout case

return None

👉 No input detected

# 🧩 2. scanner.py (BARCODE MODULE)

## ✅ FULL CODE

import cv2

from pyzbar.pyzbar import decode

import time

def scan_barcode(timeout=10):

cap = cv2.VideoCapture(0)

start = time.time()

while time.time() - start < timeout:

ret, frame = cap.read()

if not ret:

continue

barcodes = decode(frame)

for barcode in barcodes:

data = barcode.data.decode('utf-8')

cap.release()

cv2.destroyAllWindows()

return data

cap.release()

cv2.destroyAllWindows()

return None

## 🔍 EXPLANATION

### 🔹 Open camera

cap = cv2.VideoCapture(0)

👉 0 = default USB camera

### 🔹 Capture frame

ret, frame = cap.read()

👉 Gets image from camera

### 🔹 Decode barcode

barcodes = decode(frame)

👉 Finds barcode in image

### 🔹 Extract data

data = barcode.data.decode('utf-8')

👉 Converts to readable text

### 🔹 Timeout handling

return None

👉 If nothing scanned

# 🧩 3. lcd.py (DISPLAY MODULE)

## ✅ FULL CODE

from RPLCD.i2c import CharLCD

import time

lcd = CharLCD('PCF8574', 0x27)

def lcd_display(message):

lcd.clear()

lcd.write_string(message[:16])

def lcd_clear():

lcd.clear()

## 🔍 EXPLANATION

### 🔹 Initialize LCD

CharLCD('PCF8574', 0x27)

👉 0x27 = I2C address

### 🔹 Display function

lcd.write_string(message[:16])

👉 16 char limit

### 🔹 Clear screen

lcd.clear()

# 🧩 4. database.py (DATABASE MODULE)

## ✅ FULL CODE

import mysql.connector

def connect_db():

try:

return mysql.connector.connect(

host="localhost",

user="root",

password="YOUR_PASSWORD",

database="library_db"

)

except:

return None

## 🔍 EXPLANATION

### 🔹 Connect DB

mysql.connector.connect()

👉 Opens MySQL connection

### 🔹 Error handling

except:

return None

👉 Prevent crash

# 🧩 5. HOW ALL MODULES CONNECT (VERY IMPORTANT)

## 🔹 Inside main.py

from sensors import detect_mode

from scanner import scan_barcode

from lcd import lcd_display

from database import connect_db

## 🔁 FLOW

Sensor → Scanner → Database → LCD

# 🔥 COMPLETE FLOW (REAL UNDERSTANDING)

## 🔹 Step-by-step

- detect_mode()
👉 ISSUE / RETURN

- scan_barcode()
👉 Book ID

- scan_barcode()
👉 User ID

- connect_db()
👉 Connect database

- lcd_display()
👉 Show result

# ⚠️ COMMON MISTAKES (VERY IMPORTANT)

## ❌ Wrong file name

👉 sensors.py ≠ sensor.py

## ❌ Wrong import

from sensor import detect_mode ❌

## ❌ Not in same folder

## ❌ Wrong password in DB