# 💻 🧠 STEP 3: RASPBERRY PI SETUP (FULL DETAILED GUIDE)

# 📦 1. WHAT YOU NEED BEFORE STARTING

### 🔹 Hardware Required

- Raspberry Pi 4 Model B

- Micro SD Card (16GB+ recommended)

- Card Reader

- Power Adapter

- Monitor + Keyboard (or SSH)

# 💾 2. INSTALL OS (FROM ZERO)

## 🔹 STEP 2.1: Download Tool

Download:
👉 Raspberry Pi Imager

Install it on your laptop.

## 🔹 STEP 2.2: Insert SD Card

- Put SD card into laptop using card reader

## 🔹 STEP 2.3: Open Raspberry Pi Imager

Click options:

### 1. CHOOSE OS

Select:

Raspberry Pi OS (64-bit)

### 2. CHOOSE STORAGE

Select your SD card

### 3. ADVANCED SETTINGS (VERY IMPORTANT ⚠️)

Press:

Ctrl + Shift + X

Enable:

✔ Set hostname → library-pi

✔ Enable SSH → YES

✔ Username → pi

✔ Password → raspberry (or your own)

✔ Configure WiFi (if needed)

✔ Set locale → India

### 4. WRITE OS

Click:

WRITE

👉 Wait 5–10 minutes

# 🔌 3. FIRST BOOT (VERY IMPORTANT)

## 🔹 STEP 3.1: Insert SD Card into Pi

Then connect:

- HDMI (monitor)

- Keyboard

- Power cable

## 🔹 STEP 3.2: Boot Screen

You will see Raspberry Pi loading screen.

## 🔹 STEP 3.3: Login

Username: pi

Password: raspberry

# 🌐 4. INTERNET SETUP

## 🔹 Check WiFi

ping google.com

👉 If replies → internet OK

## ❌ If not working

sudo raspi-config

Go to:

System Options → Wireless LAN

Enter:

- WiFi Name

- Password

# ⚙️ 5. UPDATE SYSTEM (VERY IMPORTANT)

sudo apt update

sudo apt upgrade -y

👉 This avoids future errors

# 🔧 6. ENABLE IMPORTANT INTERFACES

## 🔹 Open Config Tool

sudo raspi-config

## 🔹 Enable THESE (VERY IMPORTANT)

Go to:

Interface Options →

Enable:

| Option | Why |
| --- | --- |
| I2C | LCD |
| Camera | Barcode scanning |
| SSH | Remote access |

## 🔹 Save & Exit

Reboot:

sudo reboot

# 📷 7. TEST CAMERA (USB CAMERA)

## 🔹 Check Detection

ls /dev/video0

👉 Output should be:

/dev/video0

## 🔹 Test Camera

import cv2

cap = cv2.VideoCapture(0)

print(cap.isOpened())

👉 Should print:

True

# 🔌 8. TEST I2C (LCD COMMUNICATION)

## 🔹 Install Tools

sudo apt install i2c-tools -y

## 🔹 Scan I2C Devices

i2cdetect -y 1

## 🔹 Expected Output

0x27 or 0x3F

👉 This is your LCD address

# 🔴 9. GPIO TEST (IR SENSOR PREP)

## 🔹 Install GPIO

sudo apt install python3-rpi.gpio -y

## 🔹 Test GPIO

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM)

GPIO.setup(17, GPIO.IN)

print("GPIO Ready")

# 🧪 10. FINAL SYSTEM CHECK SCRIPT

Create file:

nano setup_test.py

Paste:

import cv2

import RPi.GPIO as GPIO

# Camera Test

cap = cv2.VideoCapture(0)

print("Camera Working:", cap.isOpened())

# GPIO Test

GPIO.setmode(GPIO.BCM)

GPIO.setup(17, GPIO.IN)

print("GPIO Working")

print("Setup Completed Successfully!")

Run:

python3 setup_test.py

# ⚠️ COMMON ERRORS + FIXES

## ❌ Camera Not Found

👉 Fix:

- Replug USB

- Try different port

## ❌ I2C Not Showing

👉 Fix:

sudo raspi-config

Enable I2C

## ❌ Permission Error

👉 Run:

sudo python3 file.py

## ❌ Internet Not Working

👉 Reconfigure WiFi

# 🧠 FINAL CHECKLIST ✅

Before moving forward:

✔ OS installed
✔ Internet working
✔ System updated
✔ I2C enabled
✔ Camera detected
✔ GPIO working